LGP Simulator Usage

31-Mar-2011

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2011, written by Robert M Supnik

Copyright (c) 1993-2008, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	LGP Features	3

2.1	CPU	3

2.2	Typewriter Input (TTI)	5

2.3	Typewriter Output (TTO)	6

2.4	High Speed Paper-Tape Reader (PTR)	6

2.5	High Speed Paper-Tape Punch (PTP)	7

3	Symbolic Display and Input	8

4	Character Set	9

This memorandum documents the LGP-30 simulator.

# Simulator Files

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_timer.c

sim_tmxr.c

sim/lgp/		lgp_defs.h

lgp_cpu.c

lgp_stddev.c

lgp_sys.c

# LGP Features

The LGP is configured as follows:

device names	simulates

CPU			LGP-30 or LGP-21 CPU with 4096 words of memory

TTI			Typewriter input (keyboard and reader)

TTO			Typewriter output (printer and punch)

PTR			high-speed paper tape reader

PTP			high-speed paper tape punch

The LGP simulator implements the following unique stop conditions:

- LGP-30 only: arithmetic overflow

- LGP-21 only: reference to undefined I/O device.

The LOAD and DUMP commands are not implemented.

## CPU

The CPU implements either the LGP-30 or the LGP-21:

SET CPU LGP30			set LGP-30

SET CPU LGP21			set LGP-21

The default is the LGP-30.  Memory size is fixed at 4096 words.  Although memory is 32b wide, bit<31> (the low-order bit) is always zero.

The following commands implement various front panel functions:

D A value				equivalent to the MANUAL INPUT button

SET CPU FILL{=value}		equivalent to the FILL INSTRUCTION button;

if no value is given, fills IR from A;

else, fills IR from the specified value

SET CPU EXEC{=value}		equivalent to the EXECUTE button;

if no value is given, executes the

instruction in IR; else, executes the

instruction specified by the value

SET CPU MANUAL			equivalent to setting the MANUAL INPUT

switch on the Typewriter; Typewriter input

is taken from the keyboard

SET CPU TAPE			equivalent to clearing the MANUAL INPUT

switch on the Typewriter; Typewriter input

is taken from the Typewriter paper tape

reader

The following commands control the display of information:

SET CPU LGPHEX			numeric displays use LGP hexadecimal

encoding

SET CPU STANDARD			numeric displays use standard hexadecimal

SET CPU TRACK			symbolic addresses are ttss, where tt =

track (0-63) and ss = sector (0-63)

SET CPU NORMAL			symbolic addresses are normal linear

addresses,	from 0 to 4095.

The defaults are STANDARD hex and TRACK addresses.

The LGP-30 implements the following additional commands:

SET CPU 4B				sets the CPU to 4-bit input mode

SET CPU 6B				sets the CPU to 6-bit input mode

SET CPU INPUT=TTI			sets the CPU to read from the Typewriter

SET CPU INPUT=PTR			sets the CPU to read from the high-speed

reader

SET CPU OUTPUT=TTO		sets the CPU to output to the Typewriter

SET CPU OUTPUT=PTP		sets the CPU to output to the high-speed

punch

The defaults are 4-bit input mode, input and output assigned to the Typewriter.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			12		counter

A			32		accumulator

IR			32		instruction register

OVF			1		overflow flag (LGP-21 only)

TSW			1		transfer switch

BP32			1		breakpoint 32 switch

BP16			1		breakpoint 16 switch

BP8			1		breakpoint 8 switch

BP4			1		breakpoint 4 switch

INPST			1		input pending flag

INPDN			1		input done flag

OUTST			1		output pending flag

OUTDN			1		output done flag

WRU			8		interrupt character

## Typewriter Input (TTI)

The Typewriter input consists of two units: the keyboard (unit 0) and the paper-tape reader (unit 1).  The keyboard is permanently associated with the console window.  The paper-tape reader can be attached to a disk file.  The RPOS register specifies the number of the next data item to be read.  Thus, by changing RPOS, the user can backspace or advance the reader.

The Typewriter input has the following options:

SET TTI1 ASCII			default tape file format is ASCII-encoded

Flex

SET TTI1 FLEX			default tape file format is transposed Flex

SET TTI1 CSTOP			reader recognizes conditional stop

SET TTI1 NOCSTOP			reader ignores conditional stop

SET TTI RSTART			start the reader; equivalent to the START

READER lever

SET TTI RSTOP			stop the reader; equivalent to the STOP

READER lever

SET TTI START			send START signal to the CPU; equivalent

to the START COMPUTE lever

Transposed Flex has the tape channels in this order: 6-1-2-3-4-5.

The ATTACH command recognizes two switches:

ATT -A TTI1 <file>		file format is ASCII-encoded Flex

ATT -F TTI1 <file>		file format is transposed Flex

The Typewriter input implements these registers:

name			size		comments

BUF			6		data buffer

RDY			1		data ready flag

KPOS			32		count of keyboard characters

RPOS			32		position in the reader input file

TIME			24		time between keyboard polls or reader

characters

STOP_IOE		1		stop on I/O error

Error handling for the Typewriter paper-tape reader is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

## Typewriter Output (TTO)

The Typewriter output consists of two units: the printer (unit 0) and the paper-tape punch (unit 1).  The printer is permanently associated with the console window.  The paper-tape punch can be attached to a

disk file.  The PPOS register specifies the number of the next data item to be written.  Thus, by changing PPOS, the user can backspace or advance the punch.

The Typewriter output has the following options:

SET TTO1 ASCII			default tape file format is ASCII-encoded

Flex

SET TTO1 FLEX			default tape file format is transposed Flex

SET TTO1 FEED=n			punch 'n' feed (0) characters

Transposed Flex has the tape channels in this order: 6-1-2-3-4-5.  The default is ASCII-encoded Flex.

The ATTACH command recognizes two switches:

ATT -A TTO1 <file>		file format is ASCII-encoded Flex

ATT -F TTO1 <file>		file format is transposed Flex

The Typewriter output implements these registers:

name			size		comments

BUF			6		data buffer

UC			1		upper case flag

TPOS			32		count of output characters

PPOS			32		position in the punch output file

TIME			24		time from I/O initiation to completion

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

## High Speed Paper-Tape Reader (PTR)

The paper tape reader (PTR) reads data from or a disk file.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

The paper-tape reader has the following options:

SET PTR ASCII			default tape file format is ASCII-encoded

Flex

SET PTR FLEX			default tape file format is transposed Flex

Transposed Flex has the tape channels in this order: 6-1-2-3-4-5.  The default is ASCII-encoded Flex.

The ATTACH command recognizes two switches:

ATT -A PTR <file>			file format is ASCII-encoded Flex

ATT -F PTR <file>			file format is transposed Flex

The paper tape reader implements these registers:

name			size		comments

BUF			6		last data item processed

RDY			1		data ready flag

POS			32		position in the input file

TIME			24		time from I/O initiation to completion

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

## High Speed Paper-Tape Punch (PTP)

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch has the following options:

SET PTP ASCII			default tape file format is ASCII-encoded

Flex

SET PTP FLEX			default tape file format is transposed Flex

SET PTP FEED=n			punch 'n' feed (0) characters

Transposed Flex has the tape channels in this order: 6-1-2-3-4-5.  The default is ASCII-encoded Flex.

The ATTACH command recognizes two switches:

ATT -A PTP <file>			file format is ASCII-encoded Flex

ATT -F PTP <file>			file format is transposed Flex

The paper tape punch implements these registers:

name			size		comments

BUF			6		last data item processed

POS			32		position in the output file

TIME			24		time from I/O initiation to completion

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

# Symbolic Display and Input

The LGP simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as character (tape files only)

-h			display as standard hexadecimal

-l			display as LGP hexadecimal

-m			display instruction mnemonics

-n			display addresses in normal format

(overrides SET CPU TRACK)

-t			display addresses as track/sector

(overrides SET CPU NORMAL)

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		Flex character

- or opcode		instruction mnemonic

numeric		hexadecimal number

LGP hexadecimal differs from standard hexadecimal in the characters used for digits 10-15

digit			standard hex	LPG hex

10			A			F

11			B			G

12			C			J

13			D			K

14			E			Q

15			F			W

There is only instruction format:

{-}op address

'op' is always a single letter.  A track/sector address (specified by SET CPU TRACK or switch -t) is two decimal numbers between 0 and 63, representing the track and sector.  A linear address (specified by

SET CPU NORMAL or switch -n) is one decimal number between 0 and 4095.  For example:

sim> d -n 64 10640

sim> ex -mn 64

64:	B 400

sim> ex -mt 100

0100:	B 0616

# Character Set

The LGP Typewriter was a Friden Flexowriter.  Input was always upper case; output could be either upper case or lower case.  The following table provides equivalences between LPG Typewriter coding and ASCII.

Typewriter	Input			LC output		UC output

code (hex)

00		illegal		illegal		illegal

01		z or Z		z			Z

02		0 or )		0			)

03		space			space			space

04		illegal		lower case		lower case

05		b or B		b			B

06		1 or L		1			L

07		- or _		0			_

10		illegal		upper case		upper case

11		y or Y		y			Y

12		2 or *		2			*

13		+ or =		+			=

14		illegal		color shift		color shift

15		r or R		r			R

16		3 or "		3			"

17		; or :		;			:

20		newline		newline		newline

21		i or I		i			I

22		4 or ^		4			^

23		/ or ?		/			?

24		illegal		backspace		backspace

25		d or D		d			D

26		5 or %		5			%

27		. or ]		.			]

30		tab			tab			tab

31		n or N		n			N

32		6 or $		6			$

33		, or [		,			[

34		illegal		illegal		illegal

35		m or M		m			M

36		7 or ~		7			~

37		v or V		v			V

40		' (cond stop)	'			'

41		p or P		p			P

42		8 or #		8			#

43		o or O		o			O

44		illegal		illegal		illegal

45		e or E		e			E

46		9 or (		9			(

47		x or X		x			X

50		illegal		illegal		illegal

51		u or U		u			U

52		f or F		f			F

53		illegal		illegal		illegal

54		illegal		illegal		illegal

55		t or T		t			T

56		g or G		g			G

57		illegal		illegal		illegal

60		illegal		illegal		illegal

61		h or H		h			H

62		j or J		j			J

63		illegal		illegal		illegal

64		illegal		illegal		illegal

65		c or C		c			C

66		k or K		k			K

67		illegal		illegal		illegal

70		illegal		illegal		illegal

71		a or A		a			A

72		q or Q		q			Q

73		illegal		illegal		illegal

74		illegal		illegal		illegal

75		s or S		s			S

76		w or W		w			W

77		illegal		illegal		illegal

Certain characters on the Flexowriter keyboard don't exist in ASCII.  The following table provides ASCII substitution characters for the unique Flexowriter characters (this is compatible with the coding in the LGP30

paper tape archive):

Typewriter	Flex			ASCII

Code (hex)

UC 12		delta			^

UC 1E		pi			~

UC 22		sigma			#

Certain Flexowriter codes have no character equivalent of any kind.  For paper-tape reader and punch files, these are encoded as !dd, where dd is a decimal number between 0 and 63.