Data base tables "Geo Data German Houses"

Description

House Coordinates or also called Geo Coded Building Addresses are meaning the connection between the postal addresses of buildings or building complexes and their accurate positions in the coordinate system. The house coordinates are the basis for applications, which are developed to assign buildings or building complexes as exact as possible positions in the form of coordinates. By Geo Coding buildings or building complexes are cartographically representable.

Geo data with house coordinates of the Federal States

For the area of the Federal Republic of Germany house coordinates with a coverage of 100% are available. The house coordinates are delivered as a standard for the whole Federal Republic of Germany or divided in its Federal States. The approximate data scopes of the house coordinates of all German Federal States are specified here.

HoSWH  Schleswig-Holstein              736 Tsd. Buildings

HoHHA  Hamburg                         231 Tsd. Buildings

HoNDS  Lower Saxony                   2.17 Mio. Buildings

HoHBR  Bremen                          146 Tsd. Buildings

HoNRW  North-Rhine Westphalia         4.02 Mio. Buildings

HoHES  Hesse                          1.37 Mio. Buildings

HoRPL  Rhineland-Palatinate           1.17 Mio. Buildings

HoBAW  Baden-Wurttemberg              2.54 Mio. Buildings

HoBAY  Bavaria                        2.98 Mio. Buildings

HoSAL  Saarland                        313 Tsd. Buildings

HoBER  Berlin                          362 Tsd. Buildings

HoBRB  Brandenburg                     663 Tsd. Buildings

HoMVP  Mecklenburg-Western Pomerania   342 Tsd. Buildings

HoSAC  Saxonia                         825 Tsd. Buildings

HoSAN  Saxonia-Anhalt                  586 Tsd. Buildings

HoTHU  Thuringia                       577 Tsd. Buildings

HoGER  Germany (all Fed. Countries)   19.0 Mio. Buildings

Quality of the Geo Data

The geo data offered here are in high-precision quality and are present in different coordinate and reference systems. They are constantly updated by an internationally active geo data manufacturer and they are subject to a continuous quality control. The house-exact geo data are used in many mobile navigation systems. They were determined by building-inspection and partially by interpolation of exact street-sections and residential district coordinates. The coordinates are not based on information of the land registry offices or the land surveying authorities. The accuracy refers to individual buildings or building complexes. House number suffixes and house number ranges are not dissolved, since the local proximity of a building complex can be represented regularly by an individual numeric house number.

Conversion to the necessary data format

First the data base tables are present in a standard data base format (dBase, ASCII / OEM character set). They can be imported directly into MS-EXCEL, MS-ACCESS and in Borlands dBase and other programs supporting this data base format.

The freeware program CONVERT, downloadable from the site http://www.killetsoft.de/p_cona_e.htm, converts dBase formatted data base tables into other data formats with the necessary characteristics and selections. With the program for example dBase data can be converted into the SDF format (Simple Document Format) or into the CSV format (Comma Separated Value). For the use of the data on different platforms it is possible to select between the character sets ASCII / OEM  and ANSI / WINDOWS. Thus the import of the data in any data base management system or file system will be possible.

For the import in MySQL or SQL data bases the necessary "CREATE TABLE" script can be generated. Further the selection of the data on data fields and data records is possible. In addition the data can be sorted on base of the data fields. Data from several files can be joined to a common file.

Please contact us, if you need the data in another format, sort sequence or in another coordinate system.

Coordinate systems and Reference systems

The geo references of all objects are contained in the tables as geographic coordinates in degree and degree/minute/second notation, as Gauss-Krueger coordinates and as UTM coordinates.

UTM coordinates are globally present in 60 meridian strips with a width of 6 degree each. Gauss-Krueger coordinates are distributed on 120 meridian strips with a width of 3 degree each. In order to be able to accomplish country-wide distance calculations between the coordinates, the UTM coordinates and the Gauss-Krueger coordinates are converted country-wide to a uniform, national central meridian strip.

The geographical coordinates are present as the reference system "WGS84 (worldwide, GPS), geocentric, WGS84". The reference system WGS84 is standardized the in the year 1984 world-wide as "World Geodetic System" on the also WGS84 named ellipsoid. It is used for navigation with the American satellite navigation system GPS (Global Positioning System).

The Gauss-Krueger coordinates are present in the reference system "Potsdam-Datum (PD, DHDN), Bessel". This reference system together with Gauss-Krueger coordinates is still in use for the official topographic cartography of the FRG.

The UTM coordinates are present as the reference system "ETRS89 (Europe), geocentric, GRS80". ETRS89 is the reference system uniform for all European countries. GRS80 is the ellipsoid used for the mapping of the coordinates. ETRS89 is a geocentric (on the earth center referred) reference system, which is almost identical to the reference system WGS84.

Because WGS84 deviates only very slightly within millimeter range from the ETRS89, the direct unification of the here used coordinates with GPS data and modern maps is possible.

Distance calculation with right-angled and metric coordinates

Because UTM coordinates and Gauss-Krueger coordinates are converted to a uniform meridian strip, distances between two points can be calculated by the simple execution of the Pythagoras theorem. That has the advantage in relation to the computation with geographical coordinates (see below) that it is substantially simpler and much faster. The result is the distance between the points in meters.

Formula for the distance calculation with Gauss-Krueger coordinates:

difEast      = abs(GK_E_CENT_1 - GK_E_CENT_2)

difNorth     = abs(GK_N_CENT_1 - GK_N_CENT_2)

distance     = sqrt(difEast * difEast + difNorth * difNorth)

with

GK_E_CENT_1:   Easting of the first coordinate

GK_N_CENT_1:   Northing of the first coordinate

GK_E_CENT_2:   Easting of the second coordinate

GK_N_CENT_2:   Northing of the second coordinate

abs():         Absolute value function

sqrt():        Square root function

distance:      Distance in meters as result

Formula for the distance calculation with UTM coordinates:

difEast      = abs(UTM_E_CENT_1 - UTM_E_CENT_2)

difNorth     = abs(UTM_N_CENT_1 - UTM_N_CENT_2)

distance     = sqrt(difEast * difEast + difNorth * difNorth)

with

UTM_E_CENT_1:  Easting of the first coordinate

UTM_N_CENT_1:  Northing of the first coordinate

UTM_E_CENT_2:  Easting of the second coordinate

UTM_N_CENT_2:  Northing of the second coordinate

abs():         Absolute value

sqrt():        Square root

distance:      The result is the distance in meters

Distance calculation with geographic coordinates

Geographic coordinates are indicated in longitude and latitude. Usually longitude and latitude are represented in the degree notation, which is also called decimal notation. Geographical coordinates in the degree notation are for the distance computation better suitable than geographical coordinates in the degrees/minutes/second notation. For a distance computation the longitude and latitude of the first point (LON_DEC1, LAT_DEC1) and the longitude and latitude of the second point (LON_DEC2, LAT_DEC2) are needed. If the latitude has a minus sign, the point is on the southern earth hemisphere, otherwise on the northern earth hemisphere. If a longitude has a minus sign, the point is situated west of the Greenwich meridian, otherwise east of it. In the Federal Republic of Germany no minus signs occur, because all coordinates are on the northern earth hemisphere and east of Greenwich.

As preparation for the distance computation the longitude and latitude are converted into radians. The unit of the radian is [rad].

Lon1r     = LON_DEC1 * PI / 180

Lat1r     = LAT_DEC1 * PI / 180

Lon2r     = LON_DEC2 * PI / 180

Lat2r     = LAT_DEC2 * PI / 180

with

LON_DEC1:   Longitude of the first point in degree notation

LAT_DEC1:   Latitude of the first point in degree notation

LON_DEC2:   Longitude of the second point in degree notation

LAT_DEC2:   Latitude of the second point in degree notation

Lon1r:      Radian of the longitude of the first point

Lat1r:      Radian of the latitude of the first point

Lon1r:      Radian of the longitude of the second point

Lat1r:      Radian of the latitude of the second point

PI:         Circle constant Pi (3,14...)

Now the longitudes and latitudes of the two coordinates are so far prepared that they can be inserted into the formula for the distance computation.

distance =  r * acos[sin(Lat1r) * sin(Lat2r)

+ cos(Lat1r) * cos(Lat2r) * cos(Lon2r - Lon1r)]

with

sin():      Sinus function

cos():      Cosinus function

acos():     Arcus Cosinus function

r:          Earth equatorial radius = 6378137 meter

distance:   Distance in meters as result

Field widths and data types

Field      Width Typ Description

STREET     40    C   Designation of the road

STR_NO      4    N   House number of a building or building complex

POST_CODE   5    C   Postal zip code

TOWN       40    C   Designation of the town / city

QUARTER    40    C   Designation of the town quarter (optional)

MUNIC_CODE  8    C   Administration ID (municipality key)

LON_DEC     8    N   Geographic longitude in degree notation (WGS84)

LAT_DEC     8    N   Geographic latitude in degree notation (WGS84)

LON_GEO     8    N   Geographic longitude in degree/minute/second notation (WGS84)

LAT_GEO     8    N   Geographic latitude in degree/minute/second notation (WGS84)

GK_E_NAT    7    N   Gauss-Krueger easting (DHDN) on the natural meridian strip

GK_N_NAT    7    N   Gauss-Krueger northing (DHDN) on the natural meridian strip

GK_E_CENT   7    N   Gauss-Krueger easting (DHDN) on an uniform meridian strip

GK_N_CENT   7    N   Gauss-Krueger northing (DHDN) on an uniform meridian strip

UTM_E_NAT   8    N   UTM easting (ETRS89) on the natural meridian strip

UTM_N_NAT   7    N   UTM northing (ETRS89) on the natural meridian strip

UTM_E_CENT  8    N   UTM easting (ETRS89) on an uniform meridian strip

UTM_N_CENT  7    N   UTM northing (ETRS89) on an uniform meridian strip

Data field STREET

Designation of the street / road, where the building / building complex is placed. If in a town / municipality several times the same road designation occurs, the streets are differentiated with the entries in the fields POST_CODE and QUARTER.

Data field STR_NO

House number of a building or building complex.

Data field POST_CODE

Postal Zip Code of the postal area, in which the building / building complex is placed. If in a town a street designation is several times present, the address is differentiated with the entries in the fields POST_CODE and QUARTER.

Data field TOWN

Designation of the town / municipality in which the building / building complex is located.

Data field QUARTER

Designation of a town / municipality quarter in which the building / building complex is located. If the data field contains the designation "Center", the building is in the main quarter of the town. If in a town a street designation is several times present, the address is differentiated with the entries in the fields POST_CODE and QUARTER.

Data field MUNIC_CODE

Eight-digit Administration ID (municipality key).

Digits 1 and 2:  Key for the Federal State

01: Schleswig-Holstein

02: Hamburg

03: Lower Saxony

04: Bremen

05: North-Rhine Westphalia

06: Hesse

07: Rhineland-Palatinate

08: Baden-Wurttemberg

09: Bavaria

10: Saarland

11: Berlin

12: Brandenburg

13: Mecklenburg-Western Pomerania

14: Saxonia

15: Saxonia-Anhalt

16: Thuringia

Digit 3:          Key for the Administrative District

0: No Administrative District assigned

Digits 4 and 5:   Key for the County

00: No County assigned

Digits 6 to 8:    key for the City or a Municipality

000: County independent City

Data field LON_DEC

Geographic longitude (WGS84) of the building / building complex in degree notation.

The degree notation is also called the decimal notation. The minute and second portion of the coordinate are converted into a decimal fraction of a degree and are placed behind the comma.

As geodetic reference system "WGS84 (worldwide, GPS), geocentric, WGS84" is used. Please read the section "Coordinate and Reference Systems" for resuming information.

Geographical coordinates in degree notation are particularly suitable well for searches with Google Earth. Here is as an example an Internet URL with coordinates from the "Geo Data International Houses", which can represent the location of Killet Software Ing.-GbR point-exactly:

http://maps.google.com/maps?t=k&ll=51.397363,6.450883&spn=0.002,0.002

The first value behind the identifier "ll" (lat / lon) is the geographical latitude, then the geographical longitude follows. The shown URL can be inserted directly into the address field of the browser to represent a map cutout on the screen.

Digits 1 to 8:   Geographic longitude in degree

Data field LAT_DEC

Geographic latitude (WGS84) of the building / building complex in degree notation.

See information of the data field LON_DEC.

Digits 1 to 8:   Geographic latitude in degree

Data field LON_GEO

Geographic longitude (WGS84) of the building / building complex in degree/minute/second notation.

The degree/minute/second notation is also called the gradual notation. The degree, minutes and seconds of the geographical longitude and latitude are represented as two digits each before the comma. The decimal part of one second is placed behind the comma.

As geodetic reference system the WGS84 datum on the WGS84 ellipsoid is used. Please read the section "Coordinate and Reference Systems" for resuming information.

Digits 1 and 2:  Degree portion of the geographic longitude

Digits 3 and 4:  Minute portion of the geographic longitude

Digits 5 and 6:  Second portion of the geographic longitude

Digits 7 and 8:  Decimal fraction of a second

Data field LAT_GEO

Geographic latitude (WGS84) of the building / building complex in degree/minute/second notation.

See information of the data field LON_GEO.

Digits 1 and 2:  Degree portion of the geographic latitude

Digits 3 and 4:  Minute portion of the geographic latitude

Digits 5 and 6:  Second portion of the geographic latitude

Digits 7 and 8:  Decimal fraction of a second

Data field GK_E_NAT

Gauss-Krueger easting (DHDN) of the building / building complex on the natural meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digit 1:         Gauss-Krueger meridian strip number of the natural meridian

Digits 2 to 7:   Gauss-Krueger easting in meter

Data field GK_N_NAT

Gauss-Krueger northing (DHDN) of the building / building complex on the natural meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digits 1 to 7:   Gauss-Krueger northing in meters

Data field GK_E_CENT

Gauss-Krueger easting (DHDN) of the building / building complex on an uniform meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digit 1:         Gauss-Krueger meridian strip number of the uniform meridian

Digits 2 to 7:   Gauss-Krueger easting in meters on the meridian strip

Data field GK_N_CENT

Gauss-Krueger northing (DHDN) of the building / building complex on an uniform meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digits 1 to 7:   Gauss-Krueger northing in meters

Data field UTM_E_NAT

UTM easting (ETRS89) of the building / building complex on the natural meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digits 1 and 2:  UTM meridian strip  number of the natural meridian

Digits 3 to 8:   UTM easting in meter

Data field UTM_N_NAT

UTM northing (ETRS89) of the building / building complex on the natural meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digits 1 to 7:   UTM northing in meters

Data field UTM_E_CENT

UTM easting (ETRS89) of the building / building complex on an uniform meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digits 1 and 2:  UTM meridian strip number of the uniform meridian

Digits 3 to 8:   UTM easting in meters on the meridian strip

Data field UTM_N_CENT

UTM northing (ETRS89) of the building / building complex on an uniform meridian strip.

Please read the section "Coordinate and Reference Systems" for resuming information.

Digits 1 to 7:   UTM northing in meters

C. Killet Software Ing.-GbR

Postfach 400225

47896 Kempen

Germany

Tel 0049 / (0)2152 / 961127

Fax 0049 / (0)2152 / 961128

http://www.killetsoft.de

killet@killetsoft.de