Miscellaneous Changes to SWAT

* Change BTx instructions to display bit mask for immediate values.

In particular, the operand analysis display for BTx r/m16,r16

is extended to BTx  r/m16,imm8 (as well as the r/m32 forms).	This

change affects INT1_ESC.ASM, INT1_FNS.ASM, INT1_GRP.ASM,

SWAT_OPR.ASM, and INT1_FNS.INC.

* Handle ring transitions from privilege levels above 0 to PL0.  This

change is needed for DPMI support.  This change affects SWAT.ASM and

SWAT_OPR.ASM.

* When displaying physical addresses in the first megabyte via

Seg:Off, if the display wraps at 64KB, we fail with a page fault

(which is handled by displaying question marks).  By saving and

restoring one additional PTE in DISPPHYS, this problem is solved.

This change affects SWAT_MEM.ASM only.

* Note that EAX is clobbered by the IDTMAC macro.  This change affects

SWAT.ASM, SWAT_CMD.ASM, SWAT_INI.ASM, and SWAT_REG.ASM.

* Implement built-in command line keywords .PMIxx and .VMIxx to

extract the PM and VM interrupt handlers for interrupt xxh.  This

change affects SWAT_CMD.ASM only.

* Display TI and PL bits in selector.  This change affects

SWAT_CMD.ASM, SWAT_DTE.ASM, and SWAT_OPR.ASM.

* Ensure our hooked interrupts are within IDT limit.  In particular,

if we come up under Windows in a VM, interrupts 00h through 5Fh only

are defined.	We must be careful not to blast in any of our

interrupts (such as INT 67h) if they are beyond the IDT limit.  This

change affects SWAT_INT.INC, SWAT_INI.ASM, and SWAT_KEY.ASM.

* Let Windows handle the invalid opcode 0F FF if we're in PM as that's

another of its mode-switching tricks.  This change affects

SWAT_INT.ASM only.

* Define general routines to swap and restore locally hooked

interrupts from the IDT.  In particular, when we display the IDT, we

want to display the entries before we became active.	This change

affects SWAT_CMD.ASM and SWAT_DTE.ASM.

* Display call gate DTEs in a manner which separates the parm count

and selector fields.	This change affects SWAT_DTE.ASM only.

* Handle Page Faults in data search command.  In particular, if a

search match occurs in the first few bytes of memory (starting at

0:0), the context displayed to the left of the match wraps to be

negative (that is, just below 4GB) which might signal a Page Fault).

This change affects SWAT_SRC.ASM only.

* In ENABLE_IRQS, in case we need to clear IRQ1 with a specific EOI,

we must clear the keyboard command from the 8042 with a read from

I/O port 60h.  This change affects SWAT_KEY.ASM only.

INT1_FNS.INC	 Define save area for immediate value to be used in

BTx immediate display.

Define separate width bit definitions for BT with

either register or immediate operand.

INT1_ESC.ASM	 Set DWIDTH value to BTREG as per split of BT values

in INT1_FNS.INC.

INT1_FNS.ASM	 Initialize save area for immediate value in OPER_STR.

Fix bug in @PTR16@16 and @PTR16@32 when INT1_DEF is

not defined (wrong registers used).

Save immediate values in @IMM8, @IMM8D, @IMM8W,

@IMM16, and @IMM32.

INT1_GRP.ASM	 Change @GRP8 to support BTx immediate when operand

analysis is in effect.

SWAT_INT.INC	 Define flags to indicate which interrupts we've

hooked so we can restore those and only those.

SWAT.ASM	 Delete comment relating DI_STR to FORW_STR as they

are no longer related.

Separate parts of DI_STR based upon under what

circumstances they are valid.

Clear DR6 after skipping over a BC breakpoint.

Handle ring transitions upon entry to SWATTER in PM

based upon RPL of caller's CS.

Delete unneeded commented out code.

Replace tests of NT bit in RETFL with @LC2_TSS which

is set in SWAT_VCP.

Rename labels with NT to TSS as that's more

appropriate.

Note that EAX is clobbered by the IDTMAC macro.

Avoid tracing into INT 01h and INT 03h in PM.

Handle ring transitions upon exit from SWATTER in PM

based upon RPL of caller's CS.

In CHECK_JMPRET, handle OSP up front so we don't need

individual checks.

In CHECK_JMPRET, remove individual checks for OSP

instructions.

SWAT_CMD.ASM	 Precede STOS with S16 macro.

Note that EAX is clobbered by the IDTMAC macro.

Implement built-in keywords .PMIxx and .VMIxx.

Swap and restore locally hooked interrupts when

reading .PMIxx values.

In PARSE_EA, don't lose the PL bits for display

purposes.

SWAT_DTE.ASM	 Modify SWAPIDT macro to save the old value onto the

stack-based structure.

Modify RESTIDT macro to restore the old value from

the stack-based structure.

Define SWAP_OLDINTS and REST_OLDINTS to swap and

restore all locally hooked interrupts.

In DISP_DTE, don't clear the TI bit before displaying

the corresponding selector.

In DISP_DTE, display call gate DTEs in a manner which

separates the parm count and selector fields.

SWAT_INI.ASM	 Save maximum interrupt number based upon IDT limit.

Note that EAX is clobbered by the IDTMAC macro.

Ensure locally hooked interrupt are within IDT limit.

SWAT_INT.ASM	 If we're inside Windows in PM and the Invalid Opcode

0F FF occurs, pass it on to Windows.

SWAT_KEY.ASM	 Ensure locally hooked interrupt are within IDT limit.

Clear IRQ1 command before sending specific EOI.

In HOUSE_DOG, use more descriptive equate when

referencing the high-order bit of System Control

Port B on a MC system.

Fix bug in WAITIBUF_CLR with register assumes.  No

data references used the incorrect assumes.

In WAITIBUF_CLR, wait for memory refresh on MC

systems before reading keyboard status.

In KEY_INT08, use more descriptive equate when

referencing the high-order bit of System Control

Port B on a MC system.

SWAT_MEM.ASM	 Save and restore one additional PTE in case of 64KB

wrap in display of physical addresses.

SWAT_OPR.ASM	 Define test values for BTREG and BTIMM.

Define action routines for BTREG and BTIMM.

Check for both BTREG and BTIMM before calling BTDISP.

Display SS:SP for IRET if ring transition in PM.

Fix bug in comment (No ==> Yes).

Display SS:ESP for IRETD if ring transition in PM.

Change BTDISP to handle BTx r/m16,imm8.

In INST_OPR0E and REST_OPR0E, use DTR_STR instead of

constant 6.

In CHECK_PMTSS, don't lose the PL bits for display

purposes.

SWAT_REG.ASM	 Note that EAX is clobbered by the IDTMAC macro.

SWAT_SRC.ASM	 Handle Page Faults in data search command.